# Worksheet 3 Binary arithmetic

Task 1

Carry out the following binary sums showing your working out:

- 1012 + 1112

| Carry row: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 0 | 1 |
|  |  | 1 | 1 | 1 |
| Total |  |  |  |  |

- 101102 + 101112

|  | 1 | 0 | 1 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 0 | 1 | 1 | 1 |

- 112 + 1000012 +1012

|  |  |  |  | 1 | 1 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 1 |
|  |  |  | 1 | 0 | 1 |

- 101012 + 1110112 + 10012

Show how the following values can be stored as binary bytes within a computer system and determine the answer that would be calculated and stored:

- 1210 + 1310

| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

- 17410 + 25510

- 1910 + 6610 + 7410

Answer the following question:

- A computer has been designed to work only in single bytes of data. Describe the problem that will be encountered when carrying out the sum 011110012 + 111110012 if the answer is only allocated one byte of storage.

# Task 2 Binary multiplication

Carry out the following binary multiplications showing your working out:

- 10112 x 102

|  |  | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 0 |

- 111002 x 1012

|  |  |  | 1 | 1 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1 | 0 | 1 |

- 1100112 x 11102

|  |  |  |  | 1 | 1 | 0 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 1 | 1 | 1 | 0 |

- 101012 x 1000112

- 10101112 x 1011112

- 1110102 x 1010012

For each of the previous questions, convert the values into decimal and check they are correct.

# Task 3 Binary subtraction

Convert these decimal values into two’s complement binary bytes:

- -5010

- -12010

- 12710

- -12810

- Show that -5010 gives the same result as in (a) above using the following alternative method:

Assume the left-most bit represents -128. Start at -128 and then add the remaining values:

| -128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Carry out the following calculations in two’s complement binary bytes:

- -5010 -3010 or -5010 + -3010

| 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |

- -6610  + 3410

- -8810 - 1210

- 2210 - -1410

# Task 4 Fixed point binary

Convert these decimal values into a fixed-point binary byte where the first 4 bits represent the whole number part and the the last 4 bits represent the fractional part:

- 8.510

- 14.2510

- 0.12510

- 5.562510

- 1.937510

- 0.062510

Convert these fixed-point binary bytes to decimal where the first 3 bits represent the whole number part and the the last 5 bits represent the fractional part:

- 101010102

- 101110112

- 001110112

- 111110002

- 000111112

- 111111112