# Homework 1 Mealy machines

- Why are finite state machines considered to be abstract?	 [1]

2.	A speaker system responds to the press of a button.  On each press of the button, the volume is increased.  The volume output moves through a cycle of Off, Low, Medium, and High, Off, Low, ….  Here is an image of an FSM that describes this system.

Complete this Mealy machine diagram representing the speaker system.	[4]

3.	Here is a diagram of a Mealy machine.

(a)	Complete the state transition table based on this Mealy machine diagram. 	[6]

| Input | Current state | Output | Next state |
| --- | --- | --- | --- |
| 0 | S0 |  |  |
| 1 | S0 |  |  |
| 0 | S1 |  |  |
| 1 | S1 |  |  |
| 0 | S2 |  |  |
| 1 | S2 |  |  |

(b)	Show the state sequence for the input string 101110.	[5]

| Input | 1 | 0 | 1 | 1 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| State | S0 |  |  |  |  |  |  |
| Output |  |  |  |  |  |  |  |

(c)	What sequence of inputs did it identify by outputting a 1?	[1]

4.	Design a Mealy machine that translates an input string to its complement.  
For example, the input string 0101 should generate an output of 1010.	[3]

[Total 20 Marks]