# Worksheet 2A The Processor

Task 1

- Match the processor components with the correct descriptions:	[6]

| Arithmetic-Logic Unit |  | A single memory location in which intermediate arithmetic and logic results are stored |
| --- | --- | --- |
| System clock |  | Directs and coordinates the operation of the processor and controls the flow of data within the CPU |
| General purpose registers |  | Memory locations used to assist the operation of the processor in collecting and processing instructions and data |
| Dedicated registers |  | A continuously cycling signal that is used as a timing pulse for all processor components |
| Accumulator |  | Memory locations used to hold values and intermediate results whilst executing an instruction |
| Control Unit |  | Used to carry out the instructions received by the processor and produce the processed output |

Task 2

- An incomplete diagram of processor components is shown below including possible data stored in various registers. The address 027FFF95 contains the instruction ADD R0, R1, #10. Complete the diagram by labelling the name of each component.

Tip: Think about the contents and connections of each part.

| Memory Address Register | General Purpose Registers | Control Unit |
| --- | --- | --- |
| Memory Buffer Register | Current instruction Register | Program Counter |

[6]

In the spaces below, justify your decisions:

  - Memory Address Register: 	[1]

  - Memory Buffer Register: 	[1]

  - Program Counter: 	[1]

  - Current instruction Register: 	[1]

  - General Purpose Registers: 	[1]

  - Control Unit:	[1]