Worksheet 5: Input - output devices

Task 1 Reading barcodes

Barcode readers or scanners are used to read barcodes on supermarket products, library books, and for tracking packages during shipping, for example.

The Code-128 system can represent all alphanumeric characters. The UPC-A barcode below contains guard bars and represents numeric values only.

|  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |

Left    		Right

| Left Guard |  |  | 0 (Start) |  |  |  |  |  |  | 5 |  |  |  |  |  |  | 9 |  |  |  |  |  |  | 2 |  |  |  |  |  |  | 7 |  |  |  |  |  |  | 3 |  |  |  |  |  |  | Middle Guard |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- The grid below represents the left hand side of a standard UPC-A barcode. 
Shade in the bars according to the code patterns in the left side of the chart above.

- Each dark line in the barcode is represented by the binary value ‘1’ and each light line is represented by the binary value ‘0’. Thus, the number 7 (appearing on the left) has the following pattern and a binary representation of 0111011:

| 0 | 1 | 1 | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- |

Give the binary interpretation of the following barcode:

- Use an online barcode generator (such as www.barcode-generator.org) to create a UPC-A barcode for the product code number:

3 9 4 6 6   1 2 9 9 4

Task 2 Scanners and cameras

Two-dimensional scanners can be used to scan a text document. The resulting image of the text document can then be converted back into live text for editing within a Word Processor. This uses a process called Optical Character Recognition or OCR.

R

- Describe how a scanner can recognise a character of text using OCR technology.

A handheld digital camera includes face detection technology which can be used to recognise if a person’s face is within the frame and adjust the focus on the face.

- Explain one factor could cause the camera to fail to recognise a face in the picture?

Task 3 Operation of laser printers

A laser printer is about to print out a document.  The table below lists 12 steps that need to occur to print the document.  They are not in the correct sequential order.

Put the numbers 1 to 12 in the first column to show the correct sequence of steps.

| Order of step | Steps |
| --- | --- |
|  | As printer drum rotates, a laser beam scans across it removing the positive charge in certain areas; this leaves negatively charged areas which match the text or images to be printed |
|  | At the very end of the printing process, a discharge lamp removes the electric charge from the drum making it ready for the next page |
|  | Data sent to printer and then stored in a printer buffer |
|  | Data to be printed is first sent to the printer driver |
|  | Negatively charged sheet of paper is then rolled over the drum |
|  | Paper goes through a fuser which melts the ink making a permanent copy |
|  | Printer driver checks the status of the printer (e.g. is the printer out of toner?) |
|  | Printer driver puts data into a format that the printer can process |
|  | Printer drum is coated with positively charged toner; it only sticks to the negatively charged parts of the drum |
|  | Printing process starts by giving printer drum a positive charge |
|  | Toner on the drum now sticks to the paper producing exact copy of the text and images |
|  | To prevent paper sticking to the drum, electric charge is removed once the page has been printed |

Task 4 Radio frequency ID

A passive RFID chip inside a travel pass is used to board a bus as an electronic ticket. Complete the table to correctly place each of the steps on the diagram.

| Step | Position |
| --- | --- |
| RFID chip is able to receive updated data |  |
| Sensor pad emits radio energy |  |
| RFID chip is able to send identification data |  |
| RFID antenna picks up radio waves |  |
| RFID chip becomes energised |  |