Name:		Class:

Starter

In pairs or small groups, study the images below and discuss any components that are not made from a single material, such as metal, plastic, wood or fabric.

Task 1

Duct tape is a heavy-duty tape which consists of three materials:

- Cotton mesh (scrim)

- Outer polyethylene coating

- Rubber-based, pressure sensitive adhesive

Take a closer look at a piece of Duct tape.

What properties does it possess?

| Physical properties | Mechanical properties | Working properties |
| --- | --- | --- |

Task 2

Read the following facts comparing CFRP and GRP and answer the questions below.

- CFRP is lighter than GRP, it has a superior strength to weigh ratio

- GRP is more cost effective than CFRP although both require labour intensive ‘lay-up’ by hand

- CFRP is significantly more stable in extreme temperatures

- Long strands of carbon fibers are very difficult and expensive to manufacture but the manufacture of glass fibres is much easier

- Both materials are very strong, but CFRP is more rigid – GPR has more flex

- CFRP has higher tensile strength than GRP

Which material, out of GRP and CFRP, would be best suited to the following products 
and why?

| Product | Material choice and justification |
| --- | --- |
| Body of a drone | Material: Justification: |
| Hull of a boat | Material: Justification: |
| Helicopter blades | Material: Justification: |
| Rocket parts | Material: Justification: |

Task 3

Study the image below which shows a shopfront made from aluminium composite board and fibre cement.

Aluminium composite board

Fibre cement

Discuss the advantages of using each of these materials for shops and warehouses.