Name:		Class:	 	Mark:

Task 1

Study the diagram below which shows how to conduct a workshop test for toughness.

| Direction of hammer strike | Direction of hammer strike |  |
| --- | --- | --- |
| Tough |  | Brittle |

1.	Using notes and sketches show how you would conduct each of the tests below and explain how you will record your findings.

Malleability/ductility:

Electrical conductivity

Corrosion testing:

Task 2

Match the terms with the correct descriptions.

| Annealing |  | A post-hardening process for ferrous metals to relieve internal stresses, increasing toughness and reducing brittleness. |
| --- | --- | --- |
| Normalising |  | Heating metals and allowing them to cool slowly in order to remove internal stresses from work hardening, increasing malleability |
| Tempering |  | Heating a medium or high carbon steel and then quenching to increase hardness |
| Case hardening |  | Hardening the surface of a low carbon steel with a carbon layer allowing the metal core to remain soft |
| Hardening |  | Heating ferrous metals for a period of time before cooling slowly to remove internal stresses giving uniform strength and decreasing hardness |

Task 3

Evaluate the hardening and tempering of steel in comparison to case hardening steel.